Exploring the Networks

IT-307

Lab File

Submitted to

AMITY UNIVERSITY UTTAR PRADESH

In partial fulfilment of the requirements for the award of the degree of

Bachelor of technology

In

Computer Science & Engineering

By

Devanshu Mittal

5CSE2Y

(A2305220128)

Submitted to:

Dr. Shipra Shukla

DEPARTMENT OF AMITY SCHOOL OF ENGINEERING

AMITY UNIVERSITY UTTAR PARDESH

NOIDA (U.P.)

INDEX

| S. No. | Title | Date | Sign. |
| --- | --- | --- | --- |

Lab-1

Aim: To describe some of troubleshooting commands.

Commands:

ping A ping (Packet Internet or Inter-Network Groper) is a basic Internet program that allows a user to test and verify if a particular destination IP address exists and can accept requests in computer network administration.

-t Ping the specified host until stopped using Ctrl + C.

time shows the time taken by packet to be received by the user.

TTL is the total time to live for packet. The packet is informative only for this much intermediary devices.

ipconfig Displays all current TCP/IP network configuration values and refreshes Dynamic Host Configuration Protocol (DHCP) and Domain Name System (DNS) settings. Used without parameters, ipconfig displays Internet Protocol version 4 (IPv4) and IPv6 addresses, subnet mask, and default gateway for all adapters.

Default Gateway is the last device’s IP address through which the system is connected to the outer network.

finger A command-line utility known as “finger” is used to display all available information about the system's user.

tracert A traceroute plays a different role than other diagnostic tools, such as packet capture, which analyzes data.

pathping It combines the functionality of ping and tracert. It is used to locate spots that have network latency and network loss.

arp Address Resolution Protocol (ARP) is a procedure for mapping a dynamic IP address to a permanent physical machine address in a local area network (LAN).

hostname The identity of a host/website is identified by its name.

whoami It shows the effective user ID.

Lab-2

Aim: To construct five different topologies and configure them.

Network Topologies:

Star Topology

Mesh Topology

Ring Topology

Bus Topology

Hybrid Topology

Lab-3

Aim: Configuring the switches in a network.

Network Topology:

Configuration and Commands:

Switch>enable

Switch#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#hostname sw-Floor-1

sw-Floor-1(config)#line console 0

sw-Floor-1(config-line)#password cisco

sw-Floor-1(config-line)#login

sw-Floor-1(config-line)#end

sw-Floor-1#

%SYS-5-CONFIG_I: Configured from console by console

sw-Floor-1#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

sw-Floor-1(config)#enable secret class

sw-Floor-1(config)#exit

sw-Floor-1#

%SYS-5-CONFIG_I: Configured from console by console

sw-Floor-1#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

sw-Floor-1(config)#line vty 0 15

sw-Floor-1(config-line)#password cisco

sw-Floor-1(config-line)#login

sw-Floor-1(config-line)#end

sw-Floor-1#

%SYS-5-CONFIG_I: Configured from console by console

sw-Floor-1#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

sw-Floor-1(config)#service password-encryption

sw-Floor-1(config)#exit

sw-Floor-1#

%SYS-5-CONFIG_I: Configured from console by console

sw-Floor-1#show running-config

Building configuration...

Current configuration : 1212 bytes

!

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname sw-Floor-1

!

enable secret 5 $1$mERr$9cTjUIEqNGurQiFU.ZeCi1

!

!

!

!

!

!

spanning-tree mode pvst

spanning-tree extend system-id

!

interface FastEthernet0/1

!

interface FastEthernet0/2

!

interface FastEthernet0/3

!

interface FastEthernet0/4

!

interface FastEthernet0/5

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

line con 0

password 7 0822455D0A16

login

!

line vty 0 4

password 7 0822455D0A16

login

line vty 5 15

password 7 0822455D0A16

login

!

end

sw-Floor-1#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

sw-Floor-1(config)#banner motd #Authorize Access only!#

sw-Floor-1(config)#copy run start

Lab-4

Aim: To configure VLAN in a network.

Theory: VLAN is a group of end stations in a switched network that is logically segmented by function or application, without regard to the physical locations of the users.

Topology:

Configuration and Commands:

Switch>en

Switch#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#vlan 10

Switch(config-vlan)#name green

Switch(config-vlan)#vlan 20

Switch(config-vlan)#name orange

Switch(config-vlan)#vlan 30

Switch(config-vlan)#name pink

Switch(config-vlan)#vlan 40

Switch(config-vlan)#name blue

Switch(config-vlan)#exit

Switch(config)#interface range FastEthernet 0/1-5

Switch(config-if-range)#switchport mode access

Switch(config-if-range)#switchport access vlan 10

Switch(config-if-range)#interface range FastEthernet 0/6-10

Switch(config-if-range)#switchport mode access

Switch(config-if-range)#switchport access vlan 20

Switch(config-if-range)#interface range FastEthernet 0/11-15

Switch(config-if-range)#switchport mode access

Switch(config-if-range)#switchport access vlan 30

Switch(config-if-range)#interface range FastEthernet 0/16-20

Switch(config-if-range)#switchport mode access

Switch(config-if-range)#switchport access vlan 40

Switch(config-if-range)#exit

Lab-5

Aim: To configure the router and test the connectivity.

Theory: Router is an intelligent device which forwards the data between LANs and can also process the data.

Topology:

Configuration and commands:

Router>en

Router#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#hostname R1

R1(config)#interface g0/0/0

R1(config-if)#ip address 192.168.10.2 255.255.255.0

R1(config-if)#no shutdown

R1(config-if)#interface g0/0/1

R1(config-if)#ip address 192.168.11.2 255.255.255.0

R1(config-if)#no shutdown

R1(config-if)#interface loopback 0

R1(config-if)#

%LINK-5-CHANGED: Interface Loopback0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface Loopback0, changed state to up

R1(config-if)#ip address 192.168.12.2 255.255.255.0

R1(config-if)#no shutdown

R1(config-if)#interface loopback 0

R1(config-if)#ip address 192.168.13.2 255.255.255.0

R1(config-if)#end

R1#

%SYS-5-CONFIG_I: Configured from console by console

R1#copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

Testing the connectivity:

Lab-6

Aim: To configure VLAN in router using 802.1Q

Theory: The IEEE.802.1Q protocol is used to interconnect multiple switches and routers and for defining VLAN topologies.

Topology:

Commands:

Router>en

Router#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#interface g0/0/0

Router(config-if)#no ip address

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface g0/0/0.10

Router(config-subif)#

%LINK-5-CHANGED: Interface GigabitEthernet0/0/0.10, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0/0.10, changed state to up

Router(config-subif)#encapsulation dot1Q 10

Router(config-subif)#ip address 10.0.0.1 255.0.0.0

Router(config-subif)#exit

Router(config)#interface g0/0/0.20

Router(config-subif)#

%LINK-5-CHANGED: Interface GigabitEthernet0/0/0.20, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0/0.20, changed state to up

Router(config-subif)#encapsulation dot1Q 20

Router(config-subif)#ip address 20.0.0.1 255.0.0.0

Router(config-subif)#exit

Router(config)#

Router#

%SYS-5-CONFIG_I: Configured from console by console

Lab-7

Aim: Configuring router using IPv6 address.

Theory: IPv6 address consist of 128 bits. It is compacted to 32 hexadecimal digits. 4 such digits are grouped and separated by a colon.

Topology:

Commands:

Router>en

Router#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#interface gigabitethernet 0/0/0

Router(config-if)#ipv6 address 2001:db8:abcd:a::1/64

Router(config-if)#no shutdown

Router(config-if)#

%LINK-5-CHANGED: Interface GigabitEthernet0/0/0, changed state to up

Router(config-if)#exit

Router(config)#interface gigabitethernet 0/0/1

Router(config-if)#ipv6 address 2001:db8:abcd:b::1/64

Router(config-if)#no shutdown

Router(config-if)#

%LINK-5-CHANGED: Interface GigabitEthernet0/0/1, changed state to up

Router(config-if)#exit

Router(config)#interface serial 0/1/0

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0/1, changed state to up

Router(config-if)#ipv6 address 2001:db8:acad:5::1/64

Router(config-if)#no shutdown

%LINK-5-CHANGED: Interface Serial0/1/0, changed state to down

Router(config-if)#exit

Router(config)#

%LINK-5-CHANGED: Interface Serial0/1/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface Serial0/1/0, changed state to up

Lab-8

Aim: Configuring router using static IPv4 address.

Theory: In static routing the router must be interconnected by a serial connection. Later, the connections needed to be routed.

Topology:

Commands:

Router2>en

Router2#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Router2(config)# interface gigabitethernet 0/0/0

Router2(config-if)#ip address 192.168.11.1 255.255.255.0

Router2(config-if)#no shutdown

Router2(config-if)#

%LINK-5-CHANGED: Interface GigabitEthernet0/0/0, changed state to up

Router2(config-if)#exit

Router2(config)#interface serial 0/1/0

Router2(config-if)#ip address 10.0.0.3 255.0.0.0

Router2(config-if)#no shutdown

%LINK-5-CHANGED: Interface Serial0/1/0, changed state to down

Router2(config-if)#exit

Router2(config)#

%LINK-5-CHANGED: Interface Serial0/1/0, changed state to up

Router2(config)#interface serial 0/1/1

Router2(config-if)#ip address 11.0.0.2 255.0.0.0

Router2(config-if)#no shutdown

%LINK-5-CHANGED: Interface Serial0/1/1, changed state to down

Router2(config-if)#exit

Router2(config)#

%LINK-5-CHANGED: Interface Serial0/1/1, changed state to up

Router2(config)#ip route 192.168.10.0 255.255.255.0 10.0.0.2

Router2(config)#ip route 192.168.12.0 255.255.255.0 11.0.0.3

Router2(config)#ip route 10.0.0.0 255.0.0.0 11.0.0.2

Press RETURN to get started!

Lab-9

Aim: Configuring DHCP,DNS,WEB server.

Theory: In static routing the router must be interconnected by a serial connection. Later, the connections needed to be routed.

Topology:

Commands:

Router>en

Router#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#int fa0/0

Router(config-if)#ip add 192.168.1.1 255.255.255.0

Router(config-if)#no shutdown

Router(config-if)#

%LINK-5-CHANGED: Interface FastEthernet0/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/0, changed state to up

Router(config-if)#exit

Router(config)#ip dhcp pool MY_LAN

Router(dhcp-config)#network 192.168.1.0 255.255.255.0

Router(dhcp-config)#default-router 192.168.1.1

Router(dhcp-config)#dns-server 192.168.1.10

Router(dhcp-config)#exit

Router(config)#ip dhcp excluded-address 192.168.1.1 192.168.1.10

Router(config)#

Output:

Lab-10

Aim: Identify TCP,UDP header fields and operation using a Wireshark FTP,TFTP session capture.

Theory: Transmission Control Protocol (TCP) is a standard that defines how to establish and maintain a network conversation by which applications can exchange data. User Datagram Protocol (UDP) is a Transport Layer protocol. UDP is a part of the Internet Protocol suite, referred to as UDP/IP suite. FTP (File Transfer Protocol) is a network protocol for transmitting files between computers over Transmission Control Protocol/Internet Protocol (TCP/IP). Trivial File Transfer Protocol (TFTP) is a simple lockstep File Transfer Protocol which allows a client to get a file from or put a file onto a remote host.

UDP:

TCP:

| Internal Assessment (Mandatory Experiment) Sheet for Lab Experiment Department of Computer Science & Engineering Amity University, Noida (UP) |  |  |  |
| --- | --- | --- | --- |
| Programme | B. Tech CSE | Course Name | Exploring the Networks |
| Course Code | [IT307] | Semester | 5 |
| Student Name | Devanshu Mittal | Enrollment No. | A2305220128 |
| Marking Criteria |  |  |  |
| Criteria | Total Marks | Marks Obtained | Comments |
| Concept (A) | 2 |  |  |
| Implementation (B) | 2 |  |  |
| Performance (C) | 2 |  |  |
| Total | 6 |  |  |